Ultrasonic HC-SR04 , Bluetooth HC-05 and MPU6050 sensor Project Basic Function Report

Intro:

- This project call Bluetooth communication with the installation of Ultrasonic sensor and Gyro & Accelerometer.

- Sort of code enable the communication between an Arduino Board and Bluetooth module.

- Also it incorporates with an ultrasonic sensor to measure the distance and a gyro & accelerometer is to measure rotation in x, y and z axis.

- Arduino board receive the commands via Bluetooth and perform those action (applicable in separate Bluetooth circuit).

- Image snip below show the 2d circuit connection design consist of:

- Mpu6050 accelerometer & gyro

- HC-SR04 Ultrasonic

- LED red and yellow

- HC-05 Bluetooth

- Arduino Uno

- The Pin Connection: showed below:

- Dependencies and Library need to install before proceed to running code:

- SoftwareSerial

- Wire: For I2C communication with the MPU6050 gyrometer sensor

- MPU6050 library.

- Setup :

- Serial Communication: Initializes the Serial monitor at a baud rate of 9600 for debugging purposes.

- I2C and Gyrometer Initialization: Sets up I2C communication and initializes the MPU6050 gyrometer sensor. The connection with the sensor is tested, and the results are printed to the Serial monitor.

- Bluetooth Initialization: Configures the SoftwareSerial library for Bluetooth communication at a baud rate of 9600.

- Ultrasonic Sensor Pins: Sets the trigger and echo pins of the ultrasonic sensor as output and input, respectively.

- Gyrometer Calibration: Calibrates the gyrometer by placing the sensor in a stable position for a few seconds. During this process, the gyrometer anchor values are determined to eliminate drift.

- LED control :

- Red LED: Turns on when the distance measured by the ultrasonic sensor is less than 5cm.

- Yellow LED: Turns on when the distance measured by the ultrasonic sensor is more than 50cm.

- Enter and run the coding. Link for Arduino full code from my GitHub account akimaziz:

HC-SR04-HC-05-MPU6050-Project/gyro_test.ino at main · akimaziz/HC-SR04-HC-05-MPU6050-Project (github.com)

- Image from my  smartphone below showed how the setup on the breadboard :

- Next image show the application  install in android link with the data from the Arduino Serial Monitor:

- This application call as Bluetooth Terminal HC-05 can represent the result of distance and gyrometer readings.